Service manual just in case your wheel may be different then this generic info.

Torque wrench.

Dial indicator.

Necessary axle shims in .002 inch, .004 inch, .008 inch, .016 inch and .032 inch.

Wheel bearing grease.  (See note 1)

Axle nut torque can affect end play by up to .004 inch.

Do not repack the wheel bearing before checking end  play.

Torque the axle nut to 60 pounds of torque, or to the torque specified in the service manual for your motorcycle if different, with a hex socket and a torque wrench after reinstalling the wheel bearings, axle and spacers.

Mount a magnetic dial indicator on the front fender, forks or brake caliper so you can measure the lateral movement of the axle in the wheel.

Push and pull the wheel along the lateral axis of the axle while reading the measurements on the dial indicator. If the end play is greater than .018 inches the wheel bearing must be shimmed.

Remove the axle nut, axle, spacer and wheel bearing assembly with a socket wrench and socket if necessary. Insert an axle shim of one of the five commonly available thickness, between .002 and .032 inches, on the axle next to the spacer if necessary and replace the wheel bearing assembly.

Re-tighten the axle nut to 60 pounds of torque, or to the torque specified in the service manual for your motorcycle if different, with a hex socket and a torque wrench.

Re-measure the axle end play with the magnetic dial indicator. Repeat as necessary. Repack the wheel bearings with  grease and continue wheel assembly when the end play measures between .004 and .018 inches.

Note 1.   Always clean bearing in mineral sprits and dry.   Then put a walnut size ball of grease on a rubber gloved hand.   Then using a pressing motion, walk the bearing around making sure it is completely filled with grease.  Then carefully put no more then a pencil diameter snake of grease on the face that goes into the hub.  Do not over pack.