2009 Harley-Davidson Police Road King - IFB Specifications

Piston displacement of not less than 1690cc OHV Twin Cam 103
two cylinders equipped with overhead valves and featuring EITMS (Engine Idle Temperature Management
System
2 Compression ratio: 9:6:1
3 Electronic Sequential Port Fuel Injection System (ESPFI)
4 Chain Drive Twin Cams
5 Hydraulic Lifters requiring no maintenance service adjustments
B. Clutch / Transmission
1 Clutch shall be of the latest multiple nine (9) plate design, wet.
2 Transmission to be of latest design, manual type, with not less than six (6) forward speeds.
3 Clutch disengage for starter motor operation
4 Primary Drive, double-row chain, 34/46 ratio.
5 Greaseless shifter shaft with “anti-vibration” rubber spacers.
6 Gear Ratios
1st– 9.593
2nd – 6.650
3rd– 4.938
4th– 4.000
5th – 3.407
6th– 2.875
C. Brakes
Disc brakes for both front and rear wheels. Front brake having dual 11.81-inch diameter drilled discs;
rear brake shall be single disc. Brakes shall be relatively free of heat fade, with four (4) piston calipers
and uniform expanding rotors.
Note: Optional Anti-Lock Braking system available (Independent Anti-Lock System)
D. Suspension
1 Motorcycles shall be provided with a suspension system to permit optimum handling conditions as
required in law enforcement.
2 41.3 mm telescopic front forks. Rear air-adjustable shocks.
3 Rear suspension swing arm bearings shall require no lubrication for life of the motorcycle.
E. Wheels / Tires
1 Black Slotted Disc Cast Aluminum wheels, rim material to be impermeable to compressed air.
2 Tubeless type tires to have non-skid tread, designed to operate on paved highways and shall have a
full, four-ply fabric reinforcement. Bead-retention tires designed to remain on wheel during sudden
loss of pressure.
3 Sealed automotive style wheel bearings requiring no endplay adjustment and service interval of
100,000 miles (160,000 km).
F. Fuel Tank
Fuel tank to have a capacity of six (6) U.S. gallons (or 22.71 liters). Must provide a minimum of 1 gal
(3.8 L) reserve fuel capacity when main supply is exhausted.
1 Electric fuel pump.
G. Electrical System
Electrical system shall be nominal 12-volt as follows:
1 Battery. Sealed, heavy-duty maintenance-free, 12-volt, minimum rated capacity – 28 ampere hours
(Battery Council International rating), 270 cold cranking amps.
2 Ignition. Non-waste spark, alpha control.
3 Charging System. Three-phase, 50-amp high output alternator, solid state regulator.
4 Connectors. Industrial grade throughout, provided with electrical accessory connection.
5 Hand Controls. Water-resistant, integrated switches.
6 Starting System. 12-volt starter, solenoid operated engagement, relay required.
7 Horn. Sound level audible above motorcycle and traffic-generated noise.
H. Lighting
Each motorcycle shall be provided with the following lighting equipment:
1 Quartz Halogen Headlight, 55 watt low beam and 60 watt high beam.
2 Pursuit lamps front mounted PAR-36, one red and one blue (incandescent).
3 Tail light.
4 Turn Signals, self-canceling system.
5 Flashers, four-way, integrated into turn signal switches.
6 License plate lighting, two blue indicator lamps.
I. Visual Displays
1 Speedometer. Analog speedometer, range 0-120mph (or kph equivalent), (2) re-settable trip meters.
2 Odometer. Calibrated, cumulative.
3 Tachometer. Analog.
4 Fuel level gauge.
5 Emergency Light Activation Indicator.
6 Sight glass on brake master cylinders.
7 Fuel Mileage countdown till empty displayed in odometer window when Low fuel indicator lamp is lit.
8 Turn Signal Indicators
9 Neutral Indicator
10 High beam Indicator
11 Low Oil pressure Indicator
12 Engine diagnostic light
13 ABS indicator lamp (if equipped with optional ABS)
14 Security Lamp (if equipped with optional security system)
J. Feature Requirements

2. Side Stand. Jiffy stand with electronic disengage to prevent drive off with jiffy stand deployed.
3. Foot Boards. pivoting type with non-skid rubber pads.
4. Guards. Front engine guard bar and rear saddlebag guard bars.
5. Saddlebags. Factory installed, law enforcement type Injection Molded GTX saddlebags, approximately
2000 cubic inches of storage space per bag, water-resistant, utilizing police style speed latches.
6. Drive. Final belt drive, Gates Aramid fiber, reinforced Poly Chain
®
with a rubber isolated 68 tooth rear
drive sprocket.
7. Chrome, two into one into two exhaust system
8. Windshield. Constructed of Lexan
®
or clear polycarbonate, designed to break away with minimal force.
9. Mirrors. Two (2) long stem true image mirrors.
10. Key. One key fits all locks.
11. Wheelbase. 63.5 inches (1613.9 mm).
12. Laden Seat Height. 27.3 in. (693.4 mm).
13. Gross Vehicle Weight Rating. 1360 lbs. (616.8 kg.)
14. Dry Weight. 791 lbs. (358.7 kg.)
15. Running Order. 828 lbs. (375.5 kg.)
16. Service Intervals. 1
st
1000 miles and then every 5000 miles thereafter.
17. Fender Tip Running Lights. Amber at front, red at rear.
18. Warranty. 24 months, unlimited mileage